Press Release

SVTM trade show:

Temperature profiling in vacuum processes

At the SVTM vacuum technologies and materials treatment exhibition, Fluke Process Instruments presents powerful temperature profiling systems based on robust Datapaq TP3 data loggers. Featuring up to 20 thermocouple channels and an outstanding 3.6 million data points memory, the loggers perform up to 10 consecutive measurement runs. The systems are application-tailored for all kinds of heat treatment processes in primary and secondary metals production, including vacuum brazing, aluminum solution reheat, and water or gas quenching. The measured temperature range is -100…1370 °C with a ±0.3 °C accuracy. Datapaq TP3 data loggers can be pre-configured for mixed thermocouple types. They enable temperature uniformity surveys (TUS) and in-process product temperature monitoring. The Datapaq Insight analysis software includes comprehensive reporting functions. Users can easily customize reports and generate CQI-9, AMS 2750E, and ISO9000-compliant reports. Application-specific thermal barriers protect the electronics throughout the length of the process. The line-up includes, among others, special barriers for vacuum brazing furnaces with minimized out-gassing and low-height quench systems. The team will be happy to advise trade show visitors on the best way to tackle their particular application.

Fluke Process Instruments at SVTM

Lille, France, 3 – 4 July 2019

Stand 96

Illustration: Fluke Process Instruments provides application-specific thermal profilers consisting of a data logger, thermal barrier, thermocouples, and Datapaq Insight analysis software

| Illustrations: | datapaq_tp3_system | Char.s: | 1309 |
| --- | --- | --- | --- |
| File name: | 201905018_pm_datapaq_tp3_svtm_en | Date: | 06-05-2019 |

About Fluke Process Instruments

Fluke Process Instruments designs, manufactures, and markets a complete line of infrared temperature measurement and profiling solutions for industrial, maintenance, and quality control applications. Distributed worldwide under the Raytek, Ircon, and Datapaq brands, the products reflect the combined experience of over 125 years in manufacturing the world’s finest temperature measurement tools and devices.

About Fluke

Founded in 1948, Fluke Corporation is the world leader in compact, professional electronic test tools. Fluke customers are technicians, engineers, electricians, and metrologists who install, troubleshoot, and manage industrial, electrical, and electronic equipment and calibration processes.

| Contact: ## Fluke Process Instruments GmbH Sabrina Balkow Blankenburger Str. 135 13127 Berlin Germany Phone: +49 . 30 . 478 008-0 Email: marcom1.emea@flukeprocessinstruments.de Internet: www.flukeprocessinstruments.com Contact for Americas and Australasia: Lindsay Schutter lindsay.schutter@fluke.com Contact for Asia (except China): Jay Jutla jay.jutla@fluke.com Contact for China: Grace Cheng chen.cheng@fluke.com | gii die Presse-Agentur GmbH Immanuelkirchstr. 12 10405 Berlin Germany Phone: +49 . 30 . 538 965-0 Email: info@gii.de Internet: www.gii.de |
| --- | --- |